Analysis protocol BRCA1-like ness with P376 MLPA assay in Excell 2007

This document describes how you can normalize your raw data for BRCA1-like MLPA classification.

- Perform fragment analysis in Genemapper, or a similar program. Output should be a text document with a table, containing all peak areas (48 peaks per sample) for all samples, including reference samples and control samples. Check if the height of the peaks is of the same height as described by MRC-Holland in the general MLPA analysis protocol. Check also the heights of the quality control peaks.

- Open your genotype table in excel. Probes with length 255, 445 and 500 are of insufficient quality and should be removed from the table for subsequent analysis. Column A contains the sample names, column B until AT contain the subsequent probes.

- Perform the first normalization step, which is to sum all probes over the columns per sample. Column AU contains now the sum. The raw data values should now be divided by the sample sum. Column AV until CN contain the probe values divided by the sample sun (figure 1).

- Perform the second normalization step. Calculate the average of all reference samples, after the first normalization step (minimal 3 reference samples per experiment, we use Promega female reference DNA).  Divide the normalized samples from step 3 by this average reference signal. The normalized data are now in column CO until EG (figure 1).

Figure 1. An example of a fictive dataset with 3 probes, 5 samples and 3 references.

- Order the 34 BRCA1-like probes into the following order.

Your excel sheet should be look like this. The final normalized sheet should look this. This file should be saved as an tab delimited file, which serves as the input for PAM, for BRCA1-like classification.

Please be aware that Excell 2003 converts the second row (which is empty). Preferably use Excell 2007 to convert your datafile into a text delimited file, or use an advanced text editor, to fulfill to PAM input criteria.